SIREN ALARMS LTD Unit 23 Daniels Industrial Estate, 104 Bath Road, Stroud, Glos, GL5 3TJ Tel: 01453 753723 VAT reg: 131035861

PD6662:2010 AND EN50131:2006 STANDARDS EXPLAINED

From the 1st June 2012, all new Intruder and Hold Up Alarms will be designed and installed to the EN50131:2006 standard in accordance with PD6662:2010 and BS8243 for police calling systems.

* Formerly known as Personal Attack

What changes do these standards incorporate?

PD6662:2010/BS8243:2010 standards will have implications in the following key areas:

• Risk Assessment & Design

• Arming and disarming methods

• Arming must be in two stages:

Stage one initiated inside the premises;

Stage two completed outside the premises.

• Timed exit is prohibited.

• Once the entry timer has expired, a confirmed alarm will be generated by the first activated detector off the entry route (previously two detectors were required to activate).

• Tampers

• The removal of a wall mounted device will result in a tamper signal being transmitted to the Monitoring & Alarm Receiving

Centre.

• Dedicated tamper message must be sent to the Monitoring & Alarm Receiving Centre on Grade 3 systems.

• Hold Up Alarms, PA devices and Duress

• Alarm Companies need to demonstrate they have carried out a risk assessment when recommending Hold up Alarms, these will require a separate URN.

• Confirmation of Hold up Alarms and Intervention by a Monitoring & Alarm Receiving Centre is required if the URN is at risk or has been lost.

• Systems can now be designed to Grade 3 standard even if they contain Grade 2 Radio Panic alarm devices.

• Duress codes can now be used on Grade 3 systems, but must be applied for on the URN application on an individual site basis.

• Monitoring & Police Response

• The Monitoring & Alarm Receiving Centre must apply a minimum of 120 seconds filtering to confirmed alarms to allow time for cancellation.

• Commissioning & Maintenance

• PD6662:2010 incorporates the commissioning & maintenance processes of DD263 which includes remote maintenance where applicable.

• Detection walk test LED’s must only be illuminated during walk test for Grade 3 systems. This is important as it prevents a prospective intruder from determining the area of detection.

• Control Equipment

• Grade 3 PD6662:2010 control equipment incorporates technological advancements and is physically different to PD6662:2004 hardware.

Security Grading

Why do these standards affect me?

These standards have been written and designed to satisfy APCO (Association of Chief Police Officers) and insurers that Intruder and Hold up Alarms have been designed and installed by a certified company. Any new Intruder and Hold Up system should be installed to comply with PD6662:2010 and BS8243:2010 (if the system is Police calling). Systems which now lose Police Response will only be reinstated if they comply with the new changes. In some cases this will mean a control panel upgrade.

Should I upgrade my existing system to comply with PD6662?

If your system currently complies with legacy British or European standards and operates effectively then there is no need to make any changes. If you need to make a major upgrade to your old system then the upgraded system will need to comply with PD6662:2010. This may be a requirement of your insurer.

What happens if I lose Police Response?

Any system requiring a new URN will need to be upgraded to comply with PD6662:2010. Following withdrawal (prior to deletion) of Police response the following courses of action will be required to gain reinstatement:

• Confirmed DD243 (2002/2004) or BS8243:2010 systems

• Alarm companies must be able to identify the cause of the false alarm and have taken remedial action or have an additional form of confirmation added.

or

• A three month period free of false alarm calls must be able to be demonstrated to reapply for the URN.

• All systems installed prior to DD243:2002 are designated “Unconfirmed” and will need to be upgraded to a confirmed BS8243:2010 system. Reinstatement of Police response may be achieved, without a 3 month delay, following compliance with the above.

SECURITY GRADING

| SYSTEM GRADE | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- |
| LEVEL OF RISK | LOW | MEDIUM | MEDIUM/HIGH | HIGH |
| CRIMINAL PROFILE | Opportunist thieves | Contents likely to be targeted by regular criminals with some preparation | Contents perceived high value, targeted by professional criminals who invest time in planning | Organised crime group/terrorists with resources and time to plan in detail |
| KNOWLEDGE OF ALARM SYSTEMS | Little | Limited | Conversant | Highly conversant |
| TOOLS & EQUIPMENT | Basic hand tools | General range of tools, test meter, battery drill | Comprehensive range of tools and equipment, laptop etc. | Comprehensive range of tools/ equipment and ability to substitute system components and penetration of key system components |
| SYSTEM TYPES | DIY, ‘Bell only’ systems | Minimum requirement for Police calling system. Includes option X where signalling is not required | In addition to grade 2, includes anti mask detection. Dual signalling Recommended etc. | In addition to grade 3, ability to recognise substitution of components and penetration of key system components e.g. controls, sounder |
| TYPICAL INSTALLATIONS | Domestic | Corner Shop Small Office High end Domestic | Bank, Bonded Warehouse, Mobile Phone Shop, Jewellers, Fashion Outlet, high risk offices etc. | Bullion/Cash Centre, Sensitive Research Establishment, Weapons arsenal etc. |
| INSURANCE VIEW | Not recognised | Required for some domestics and lower risk commercial premises | Required for many commercial premises | Required when security takes precedence (not all system components are available at the moment) |